- SPEC NO.: CU-002SDIP

Specification

TO:1123/1463

Model Name: Crystal Unit

PART NO: S3225-12.000M-9-20-20-A CUSTOMER PART NO.:

APPROVAL SHEET 样品承认书

| Approved? | Yes |
| --- | --- |
|  | No. |
| Customer's comments are welcomed here. Please return this copy as a certificate of your approval by email. Approved By Date: ______________ |  |

STRONG ELECTRONICS&TECHNOLOGY LIMITED

深圳市思硕电子科技有限公司

Tel:86-755-84528985    Fax: 86-755-84528986

Email:info@strongfirst.com.cn

www.strongfirst.com.cn

Strong Electronics&Technology Limited                                                                          www.strongfirst.com.cn

Crystal Unit Specification

History Record

| Date | Part No. | SPEC No. | Description. | Remarks. |
| --- | --- | --- | --- | --- |
| 2018-09-12 | S3225-12.000M-9-20-2 0-A | V1.0 | Initial issue | Xuguangdong |
| 2024-07-29 | S3225-12.000M-9-20-2 0-A | V1.0 | Marking update | Xuguangdong |
|  | - ISO9001:2000 ISO14001:2004 | Approved by | Check by | Design by |
|  |  | May-15-2007 | May-10-2005 | Jan-16-2005 |
| Versions | Total Page | Xu gang dong | Liu jun | Wang hon |
| CU-002SDIP |  |  |  |  |

1.Range:

This     specification     shall     cover     the     characteristics      of    Crystal      Unit     with     Strong’s     P/N:

S3225-12.000M-9-20-20-A

ROHS Compliant (Pb Free)

Small Seam sealed crystal units Surface Mount Package

High precision characteristic covering up to high frequency range

Higher frequency stability and reliability Tape and Reel packing available

2.Specification

| Item | Specification |
| --- | --- |
| Frequency Range | 12.000MHz |
| Frequency Tolerance (at 25 ℃) | ±20ppm |
| Frequency Stability Over Operating Temperature Range | ±20ppm |
| Operating Temperature Range | -50 ~ +105℃ |
| Shunt Capacitance | 3.0pf Max |
| Load Capacitance | 9pF |
| Drive Level | 0.01μW ~ 300μW, 100μW typical |
| Storage Temperature Range | -55~ +125℃ |
| Aging | ± 3ppm / year Max |

ESR

| Frequency | Mode | ESR |
| --- | --- | --- |
| 10.0MHz | Fundamental | 150 ohms max |
| 11.0~ 11.9MHz | Fundamental | 120 ohms max |
| 12.0~12.9MHz | Fundamental | 60 ohms max. |
| 13.0~ 15.9MHz | Fundamental | 60 ohms max. |
| 16.0~20.9MHz | Fundamental | 50 ohms max. |
| 21.0~29.9MHz | Fundamental | 60 ohms max. |
| 30.0~60.0MHz | Fundamental | 30 ohms max. |
| 72.0~ 156.25MHz | 3rd | 100 ohms max. |

3.Dimension (mm)

S3225-4 type      3.2mm*2.5mm

Equivalent Circuit：

4.  Marking

12.000MHz

5. Reliability Specification

|  | Item | Condition | Standard |
| --- | --- | --- | --- |
| 1. | Drop characteristics | Free drop from 75cm height on a hard wooden board for 3 times. (Board is thickness more than 30 mm.) | Frequency change: ≤±5ppm |
| 2 | Mechanical shock | Device are shocked to half sine wave (1000g) three mutually perpendicular axes each 3 times | Frequency change: ≤±5ppm |
| 3. | Shake characteristics | Shake frequency 10~55Hz, cyc1~2 minutes, swing 1.5mm, direction x/y/z, all 30 minutes, test after 1 hours. | Frequency change: ≤±5ppm |
| 4. | Humidity characteristics | +40±2℃ & 90%~95% R.H. 250 hours | Frequency change: ≤±5ppm |
| 5. | Low temperature characteristics | -40±2℃, 250 hours, put in room temperature, test after 1 hours. | Frequency change: ≤±5ppm |
| 6. | High temperature characteristics | +85±2℃, 250 hours, put in room temperature, test after 1 hours. | Frequency change: ≤±5ppm |
| 7. | Temperature cycling | -30±3℃/30±3 min~+85±2℃/30±3min, 5 cycles | Frequency change: ≤±5ppm |
| 8. | Refluence examination |  | Frequency change: ≤±5ppm |

6. Packing Specification

|  | 49S-SMD | S8045 | S7050 | S6035 | S5032 | S4025 | *S3225 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| W | 24.00±0.30 | 16.00±0.05 | 16.00±0.05 | 12.00±0.05 | 12.00±0.05 | 12.00±0.05 | 12.00±0.05 |
| E | 1.75±0.10 | 1.75±0.10 | 1.75±0.10 | 1.75±0.10 | 1.75±0.10 | 1.75±0.10 | 1.75±0.10 |
| F | 11.5±0.10 | 7.5±0.10 | 7.5±0.10 | 5.5±0.10 | 5.5±0.10 | 5.5±0.10 | 5.5±0.10 |
| T | 0.40±0.05 | 0.35±0.05 | 0.35±0.05 | 0.35±0.05 | 0.35±0.05 | 0.35±0.05 | 0.30±0.05 |
| P | 12.00±0.10 | 8.00±0.10 | 8.00±0.10 | 8.00±0.10 | 8.00±0.10 | 8.00±0.10 | 8.00±0.10 |
| P0 | 4.00±0.10 | 4.00±0.10 | 4.00±0.10 | 4.00±0.10 | 4.00±0.10 | 4.00±0.10 | 4.00±0.10 |
| P2 | 2.00±0.10 | 2.00±0.10 | 2.00±0.10 | 2.00±0.10 | 2.00±0.10 | 2.00±0.10 | 2.00±0.10 |
| D0 | φ1.50+0.10 | φ1.50+0.10 | φ1.50+0.10 | φ1.50+0.10 | φ1.50+0.10 | φ1.50+0.10 | φ1.50+0.10 |
| D1 | φ1.50MIN | φ1.50MIN | φ1.50MIN | φ1.50MIN | φ1.50MIN | φ1.50MIN | φ1.50MIN |
| A0 | 4.60±0.10 | 4.85±0.10 | 5.40±0.10 | 3.90±0.10 | 3.60±0.10 | 2.80±0.10 | 2.85±0.10 |
| K0 | 4.40±0.10 | 1.90±0.10 | 1.80±0.10 | 1.50±0.10 | 1. 10±0.10 | 0.90±0.10 | 0.85±0.10 |
| B0 | 14.20±0.15 | 8.60±0.15 | 7.40±0.10 | 6.40±0.10 | 5.40±0.10 | 4.30±0.10 | 3.55±0.10 |
| A | φ330±1.0 | φ178±2.0 | φ178±2.0 | φ178±2.0 | φ178±2.0 | φ178±2.0 | φ178±2.0 |
| B | 2.30±0.20 | 2.00±0.50 | 2.00±0.50 | 2.00±0.50 | 2.00±0.50 | 2.00±0.50 | 2.00±0.50 |
| C | φ13.5±0.20 | φ13.2±0.20 | φ13.2±0.20 | φ13.2±0.20 | φ13.2±0.20 | φ13.2±0.20 | φ13.2±0.20 |
| D | φ21.5±0.20 | φ20.0±0.50 | φ20.0±0.50 | φ20.0±0.50 | φ20.0±0.50 | φ20.0±0.50 | φ20.0±0.50 |
| N | φ100.0±0.5 | φ60.5±1.0 | φ60.5±1.0 | φ60.5±1.0 | φ60.5±1.0 | φ60.5±1.0 | φ60.5±1.0 |
| W1 | 24.5±0.20 | 16.5±0.20 | 16.5±0.20 | 12.5±0.20 | 12.5±0.20 | 12.5±0.20 | 12.5±0.20 |
| T1 | 2.30±0.20 | 1.80±0.20 | 1.80±0.20 | 1.80±0.20 | 1.80±0.20 | 1.80±0.20 | 1.80±0.20 |

© 2009 StrongFirst. All Rights Reserved.